DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

Subject: EE6303- Linear Integrated Circuits & Applications

UNIT I IC FABRICATION

PART A

- Write the chemical reaction in the epitaxial growth process of pure Silicon

- What are the advantages of Integrated circuits over discrete component circuits?

- Explain why Inductors are difficult to fabricate in IC’s

- List out the reaction compounds and doping hydrides used in Chemical Vapour Deposition?

- What is Ion Implantation? Why it is preferred over diffusion process?

- What are the different levels of Integration?

- Give classification of Integrated circuits.

- What do you mean by monolithic process?

- Explain importance of SiO2 layer.

- Name the parameters which govern the thickness of the film in the oxidation process

- Compare Dry oxidation and Wet oxidation

- What is the step taken to avoid material defect in Ion Implantation?

- What is meant by dielectric isolation in IC fabrication?

- Name different types of IC packages

- What is the need of buried layer in fabrication of monolithic integrated transistor?

- What is meant by parasitic capacitance in an IC?

- Name different methods used in fabrication of Integrated Resistors?

- Define an Integrated circuit

- What is Epitaxial growth?

- What is meant by Planar technology?

- What are the limitations of Integrated circuits

- Differentiate thin film and thick film technology in IC fabrication

- What are the different linear IC packages?

PART B

- Describe in detail any two isolation technique used to provide isolation between various components in IC fabrication with illustrations?

- Explain in step by step basis, the fabrication of Planar P-N Junction Diode with neat illustrations

- Describe the Epitaxial growth process and photolithography process with neat diagram

- Give the various ways for making Integrated Resistor

- Explain in detail about the processing steps involved in the fabrication of monolithic Integrated Circuit

- Describe in detail about the Oxidation  process of IC fabrication

- Explain the process of Crystal growth and Wafer preparation in detail

- Describe in detail about the diffusion process of IC fabrication

- Explain in detail about Ion Implantation and Metallization in detail

- Discuss the methods used for fabricating IC Resistors, Capacitors and Inductors

- Completely describe in detail the various stages involved in the fabrication of R, C and transistor in a single chip

- Explain the basic Planar processes in IC fabrication with neat diagrams

- Explain the fabrication process of Transistors and FET’s

UNIT II CHARACTERISTICS OF OPAMP

PART A

- What is the input impedance of a non-inverting amplifier?

- What are the different kinds of packages of IC 741?

- Mention the important ideal characteristics of an OP AMP.

- Draw the frequency response characteristic of an AC Integrator and indicate the part where it behaves as a True Integrator?

- State the causes for Slew rate in an operational amplifier? How it is indicated?

- Define CMRR? State its ideal value

- What is an operational amplifier?

- What is input offset voltage?

- What is input bias current and input offset current?

- What is PSRR? What should be its ideal value?

- What is slew rate? What causes the slew rate? State slew rate equations.

- Draw the typical frequency response of the op-amp

- Why frequency compensation is required?

- Mention linear and non-linear applications performed by an operational amplifier

- Compare inverting and non-inverting amplifiers

- What is an unity follower? Give its schematic and applications.

- How to obtain the average circuit from the inverting summer?

- Draw the integrator circuit using op-amp. Give its output equation.

- List the applications of Integrator circuit

- Draw the circuit diagram of an ideal differentiator. Write expression for its output.

- Why Integrators are preferred over differentiators in analog computers

- Determine the output voltage of a differential amplifier having differential gain 2000 and the CMRR 100 dB

- Give the four differential amplifier configurations

- An input of 3V is fed to the non-inverting terminal of an operational amplifier. The amplifier has Ri of 10kΩ and Rfof 10kΩ. Find the output voltage

PART B

- Explain the operation of a basic differential amplifier.

- List the six characteristics of an ideal op-amp and explain in detail. Give the practical op-amp equivalent circuit

- Explain in detail about DC characteristics of op-amp

- With a neat circuit diagram and equations, discuss the operation of summer and differentiator using operational amplifier

- Explain the AC characteristics of an operational amplifier

- Briefly explain different types of frequency compensation techniques applied to op-amp circuits

- Deduce the expression for a closed loop voltage gain of a inverting and  non-inverting amplifier

- Explain the working of non-inverting summer with neat circuit diagram

- Draw and explain the circuit of a V to I and I to V converter

- Explain the working of Integrator and Differentiator

- Design an op-amp circuit to give an output voltage Vo=3V1-2V2+5V3, where V1,V2,V3 are inputs

- Design a practical differentiator circuit that will differentiate an input signal with maximum frequency 150 Hz

- For the non-inverting op-amp shown in Fig., find the output voltage Vo

- Determine the output voltage for the following circuit

- Determine the output voltage Vo and the current Io in the circuit as shown below

- Obtain the closed loop voltage gain Vo/Vi of the circuit shown below