QUESTION BANK

CE 2402 ESTIMATION AND QUANTITY SURVEYING

SJCET / THANJAVUR / CIVIL DEPARTMENT

UNIT-I

2 MARKS

- What are the types of estimate?

- Estimate the quantities of brickwork and plastering required in a wall 4m long, 3m high

And 30 cm thick. Calculate also the cost if the rate of brickwork is Rs.32.00 per cu.m and

of plastering is Rs. 8.50 per sq.m

- What are the main components of culvert?

- Define lead and lift

- Differentiate Floor area and carpet area?

- A cement concrete road (1:2:3) is to be constructed over the existing water bound

macadam road .The thickness of slab =10cm.The length of the road is one km and the

width 3.60m.Calculate the quality of cement concrete and the material required,

- What are the types of arches?

- Define culvert

- What is canal fall?

- What are the types of culvert?

- Briefly explain about preliminary Estimate

- Write the duties of quantity surveyor

- What are the methods of taking out quantities?

- What is prime cost

- What is schedule of rates?

- Define joineries

- Define estimate

- What is the importance of soak pit?

- List the various sanitary fittings?

- Draw and mention the parts of culvert

16 marks

- Calculate the quantities of box culvert.

a) Earth work in foundation

b) Sand filling

c) PCC in foundation

d) Brickwork cm 1:4 in foundation

d) RCC 1:2:4 cover slab

e) Brickwork cm 1:4 in parapet

- What are the various methods of estimate

- Explain in detail about

  - Joineries

  - Arch types

  - Abstract estimate

  - Supplementary estimate

- Estimate the quantities of brickwork in an arch over 1.8m span opening. the arch are 40cm thick and breadth of wall is 40cm

- Prepare the detailed estimate of two room building with front verandah from the drawing

a) BW in cm 1:5 in superstructure

b) Earth work excavation

c) Ceiling plastering cm 1:3

- What are the methods of measurement and give the units of  measurement for the following items

    - Plinth beam

    - Plastering

    - Flooring

    - Earthwork excavation

- Prepare a detailed estimate of a septic tank in the soak pit and calculate the quantities for

  - Earth work excavation

  - PCC in foundation 1:3:6

  - Brickwork cm 1:4 in septic tank

  - Brickwork cm 1:6 with second class brick in soak pit

  - Precast RCC work of roof cover slab

  - Plastering with cm 1:3 in 12mm thick

- Draw the different types of sanitary fittings?

- Calculate the quantities of all item of work for the construction of retaining wall for length of 20m.

- Calculate the quantities of following item in fig

  - First class Brickwork cm 1:6 with superstructure include parapet

  - PCC in foundation

  - Ceiling plastering

UNIT-II

- The cost of construction of an overhead water tank of capacity 100mm2 is Rs 5,75,000 find out the unit rate/lr

- Differentiate abstract and detailed estimate

- Explain the difference between soak pit and dispersion trench?

- Calculate the no of standard modular bricks required for flat bricks soling for 1km length of 3.7m wide road.

- What are factors to be considered in design of septic tank?

- Write the formula for Mid ordinate rule and Prismoidal formula Rule

- .Briefly explain about revised estimate

- What are the methods of taking out estimates?

- Briefly explain about Out to Out and in to in method.

- What is the formula for mean of the segmental arch with span and rise given?

- Give the units of measurement  a)DPC b)weathering course

- What is centerline method?

- Estimate the quantities of brickwork in an arch over 1.8m span opening.the arch are 40cm thick and breadth of wall is 40cm.

- Define lumpsum

- What is painting co efficient?

- Estimate the quantities of brickwork and plastering ina wall 4m long,3m high and 30cm thick. Calculate  the cost if the rate of brickwork is Rs.32/cu.mm and plastering Rs8.50/sq.m

- Workout the quality of stone metal required for 2Km.Length for wearing coat of a 4m

Wide road. The thickness of the metal road required is 12cm loose

- Explain Cubical Content Estimate

- What are methods to be adopted for volume calculating?

- Define aqueduct

16- Mark

- Estimate the quantities of following items of septic tank in fig

  - Earth work excavation

  - PCC in  1:5:10

  - RCC work of roof cover slab

- The plan and sectional elevation of a building are given. Estimate the following quantities of item

  - Doors and windows

  - RCC work in roof slab, lintels and sunshades

  - Plastering internal

- Estimate the quantities of following items of septic tank in fig

  - Plastering.

  - Brickwork cm 1:6.

  - RCC work for soak pit cover.

- Prepare a detailed estimate of a two room building with front verandah for the given drawing

  - Earth work excavation.

  - Ceiling plastering cm 1:6.

  - BW in cm 1:5 in foundation and basement.

- Estimate in detail the quantities of the following item of work in a building for two room size of (3.6m*4.5m) and (3.6m*4.2m) shown in fiq

  - Earth work excavation in foundation.

  - 2.5cm DPC.

  - First class Brickwork cm 1:6.

- Explain the various types of estimate.

- Explain the four types of approximate estimate?

- Explain in detail about septic tank and dispersion trench?

- Details of a R.C.C retaining wall 25m long. Prepare a detail estimate for the work.

- What are the methods of estimation of roads?