M.TECH. DEGREE EXAMINATION, December 2013

Branch: Civil Engineering

Specialization: Environmental Engineering

Model Question Paper - I

First Semester

MCEEE 105 –4 ENVIRONMENTAL GEOTECHNOLOGY (Elective I)

(Regular – 2013 Admissions)

Time : Three Hours						   Maximum : 100 marks

Instructions: 	1) Answer questions in full.

2) Assume suitable data wherever necessary.

- (a) Explain in detail the sources, production and classification of waste.            (15 marks)

(b) What are the different causes of soil pollution?                                             (10 marks)

OR

- (a) What are the factors governing soil pollutant interaction?                             (10 marks)

(b) Briefly describe the failures of foundations due to pollutants.                         (5 marks)

(c) Explain the environmental cycle and their interaction with the environment.

(10 marks)

3)  	(a) What are the factors to be considered while selecting site for landfills?          (5 marks)

(b) Explain the different methods for analyzing the stability of landfills.           (15 marks)

(c) Give notes on waste characterization.                                                               (5 marks)

OR

- (a) Explain the control measures and storage system for hazardous wastes.       (12 marks)

(b) Explain in detail organic and inorganic stabilization of waste disposal sites.

(5 marks)

(c) Explain the following terms:                                                                    (2 marks each)

- Absorption

- Adsorption

- Precipitation

- Detoxification

5)   (a) Explain in detail the contaminant transport in sub surface.                            (10 marks)

(b) Write short notes on:                                                                                (5 marks each)

- Ion exchange

- Bearing capacity of compacted landfills

- Contaminant transformation

OR

- (a) What are the hydrological factors to be considered in design of landfills?      (8 marks)

(b) Give a brief note on ground water pollution.                                                    (8 marks)

(c) Explain in detail the pollution of aquifers by mixing of liquid waste.              (9 marks)

- (a) Explain how the characterization and identification of contaminated ground soil is

done.                                                                                                                (12 marks)

(b) Explain in detail the exsitu and insitu remediation of contaminated soils.    (13 marks)

OR

- (a) Give notes on                                                                                            (5 marks each)

- Bioremediation

- Bioventing

- Soil washing

- Electrokinetics

- Soil heating