(Pages : 2 )	 Name    ……….......................

Reg. No………........................

M.TECH. DEGREE EXAMINATION

Model Question Paper - I

First Semester

Branch: Electrical Engineering

Specialization: 1. Power Electronics and Power Systems

2. Power Electronics

| MEEPP 105-3/ MMEPE 105-2 | ROBOTICS AND AUTOMATION (Elective-1) |
| --- | --- |

(2013 Admission onwards)

(Regular)

Time: Three Hours						          Maxm. 100 Marks

- (a) Describe the essential features of i) Point-to-point robotic systems			[07]

- Continuous-path robotic systems			[08]

(b) Classify robots on the basis of their coordinate systems. 					[10]

OR

2	(a)  Sketch a 6-DOF Cartesian robot. Briefly describe its features.				[15]

(b) List out the essential components of a robot vision system, its advantages and appns.   [10]

3. 	(a) Determine the translated vector  for the vector v=25i + 10j + 20k when translation

is performed by 8 in the x direction, 5 in the y direction and 0 in the z direction. 	            [10]

(b) Describe the Inverse kinematics problem. What are its characteristics ?			[10]

OR

4. 	(a) Derive the values of the joint angles for a two-link system, having link dimensions l1

and l2. The coordinate of the end-effector is (x,y) for a 2-dimensional work area. 	            [10]

(b) Explain the Denavit-Hartenberg method of transformation. 				[15]

5. 	(a) Determine the inertia tensor of a rectangular body of uniform density  in the context

of a Rectangular coordinate system.								[13]

(b) Outline robot arm dynamics, emphasizing the forward and reverse dynamics cases.      [12]

OR

6. 	(a) Describe the three Eulerian angle representation of a rigid body rotation.		[13]

(b) State the Lagrange-Euler equation. Describe all the terms in detail.    			[12]

7. 	(a) Detail the role and necessity for interpolators for trajectory planning. 			[13]

(b) Describe the trajectory planning algorithm.						[12]

OR

8. 	(a) Describe four robotic joints in detail.							[13]

(b) What are the common constraints for planning joint-interpolated trajectories?		[12]

-***-