(Pages : 2 )	 Name    ……….......................

Reg. No………........................

M.TECH. DEGREE EXAMINATION

Model Question Paper - I

First Semester

Branch: Electrical Engineering

Specialization: Power Electronics and Power Systems

| MEEPP 106-3 | HIGH VOLTAGE DC TRANSMISSION (Elective-1) |
| --- | --- |

(2013 Admission onwards)

Time: Three Hours						          Maxm. 100 Marks

- (a) Explain in detail the reasons for preferring HVDC over HVDC for long distance power transmission								 			[10]

(b) How is power flow control achieved in HVDC systems.					[08]

(c) Explain the choice of DC voltage for a fixed amount of power transfer .		[07]

OR

2	(a) Draw a neat sketch of HVDC transmission scheme and explain the components           [13]		(b) Compare HVDC and HVAC based on

- Stability

- Bulk power transfer

- Stability                                                                                                          [12]

3. 	(a)  Draw the Graetz circuit and draw the timing diagram considering the overlap less than 60° and without overlap for

- Voltage across the load

- Voltage across any two conducting pair of valves                                          [15]

(b) Explain the dependency of power factor angle on firing angle (α)          		             [10]

OR

4. 	(a)  Explain the effect of overlap on the performance of converter.                                       [10]

(b) Among the different converter configurations prove that Graetz circuit is the most suited for       HVDC application 				                                                            [15]

5. 	(a) Explain in detail the reactive power requirements in HVDC terminal equipment.          [10]

(b) Why is constant current control preferred over constant voltage control                         [08]

(c) Why is pf kept high in the AC system connected to the HVDC link.                              [07]

OR

6. 	(a) Explain in detail control characteristics of Converter in HVDC system.        		[13]

(b) Explain real and reactive power control during steady state and transient conditions.    [12]

7. 	(a) Derive the expression for ripple current in the DC current. Draw the waveform of ripple voltage 	and ripple current for α=30°.                                                                                                  [13]

(b) What are the effects of Harmonics on the AC power system.       			[12]

OR

8. 	(a) What are the different scenarios where HVDC lines are subjected to over-voltages. Explain in brief the different overvoltage protection schemes.		                                                [13]

(b) Explain the importance of smoothing reactors in a DC link. How the commutation failure prevention is achieved	.	                                                                                                [12]

-***-