(Pages : 2 )	 Name    ……….......................

Reg. No………........................

M M.TECH. DEGREE EXAMINATION

Model Question Paper - II

First Semester

Branch: Electrical Engineering

Specialization: Power Electronics and Power Systems

| MEEPP 106-3 | HIGH VOLTAGE DC TRANSMISSION (Elective-1) |
| --- | --- |

(2013 Admission onwards)

(Regular)

Time: Three Hours						          Maxm. 100 Marks

- (a) What are the advantages of HVDC when compared to HVAC.  			             [13]

(b) Classify different types of HVDC systems based on :

- Construction

- Application

OR                                                                 [12]

2      (a)  How is HVDC economical when compared to HVAC for bulk power transmission.    [10]

(b)  How is power flow control achieved in HVDC systems.			 	 [08]

(c) Explain the choice of DC voltage for a fixed amount of power transfer .		 [07]

3       (a) Draw the Graetz circuit and draw the timing diagram considering the overlap greater than 60° and without overlap for

- Voltage across the load

- Voltage across any two conducting pair of valves                                               [15]

(b) Explain the dependency of power factor angle on firing angle (α)          .                       [10]

OR

4. 	(a) Derive the expression for output voltage across the load for a firing angle less than 60° in the      3-ph bridge circuit. 	            							            [08]

(b) Draw the Graetz circuit and draw the timing diagram considering the overlap less than 60° and without overlap .Also draw the waveforms of  Current in any phase . Specify the assumptions made.                              				                                                            [07]

(c) Explain Design requirements of a converter circuit. Show that 3-phase bridge configuration is the most suitable for HVDC link.                                                                                              [10]

5. 	(a) Explain constant-voltage parallel scheme and constant current series scheme for multi terminal DC lines.										             [10]

(b) Why is constant current control preferred over constant voltage control                          [08]

(c) Explain real and reactive power control during steady state and transient conditions      [07]

OR

6. 	(a) Explain in detail control characteristics of Converter in HVDC system.        		[13]

(b) Explain in detail the reactive power requirement of a converter station. How is the reactive power    supplied?                                                                                                                                 [12]

7.   (a) Derive the expression for ripple current in the DC current. Draw the waveform of ripple voltage 	and ripple current for α=30°                                                                                                   [13]

(b) What are the effects of Harmonics on the communication systems like telephone line. [12]

OR

8. 	(a) What are the different scenarios where HVDC lines are subjected to over-voltages. Explain in brief the different overvoltage protection schemes.		                                                [13]

(b) Explain in detail the DC line oscillations and the line dampers used.                              [12]

-***-