M.TECH. DEGREE EXAMINATION

Model Question Paper - I

Second Semester

Branch: Civil Engineering

Specialization: Geomechanics and Structures

MCEGS 201 GROUND IMPROVEMENT TECHNIQUES

(Regular – 2011 Admission onwards)

Time: Three Hours							        Maximum: 100 marks

Answer all questions.

- Write short notes on

- Vibro compaction piles

- Preloading methods

- Design of vertical drains

(5+10+10 marks)

OR

2.      What is a stone column? Discuss different construction techniques of stone column.

How load carrying capacity is calculated?				           (25 marks)

3.     a) Write a short note on types of grout and the desirable characteristics of a grout.			                                                                                                           (10 marks)

b) What is jet grouting? Explain the jet grouting process. Discuss the various                       application of jet grouting.						          (15 marks)

OR

4.     a) What you mean by soil stabilization? Explain the principle and mechanism of  lime  stabilization.                                                                                                (15 marks)

b) Write short note on i) Applications of lime stabilization

ii) Mechanism of cement stabilization                                     (10 marks)

5.    Explain soil reinforcement. What is the mechanism? Explain reinforcement- soil                       interaction. Mention the various types of reinforcing elements.		           (25 marks)

OR

[P.T.O]

6.  Design a reinforced earth retaining wall to retain 5 m height of fill material, which  consist                of sand γ = 18kN/m3, Ø = 40, FOS  against tension and pullout failure can be assumed to be 2. No surcharge. The wall is designed using galvanized iron metallic strip as reinforcement. The properties of  strip are as follows.

Width of strip = 10cm; thickness = 2mm; ultimate tensile strength = 24kN/m2; angle of friction between strip and soil = 28; Horizontal spacing of strips = 0.8 m; Nγ = 80		                                                                                                                        (25 marks)

7.	An 8m high wall is to be built using sand fill and polymer-grid reinforcement. The sand has Ø’ = 30, γ = 18kN/m3 and is to be used for the wall and the backfill. A surcharge loading of 15kPa is to be allowed for; the maximum safe bearing pressure for the foundation soil is 300kPa. Three grids of different design strengths are available: grid A at 24kN/m, grid B at 16.5kN/m and grid C at 10.5kNm (all having a bond coefficient fb of 0.9). The fill will be compacted in layers 250mm thick. Check the stability and design properly.

(25 marks)

OR

8.	a) Explain various types of geosynthetics. Explain the manufacturing process of geogrids.                                                                                                         (15 marks)

b)Discuss the application of geosynthetics in i) Pavement construction

ii) Environmental aspects.											          (10 marks)