M.TECH. DEGREE EXAMINATION

Branch : Electrical and Electronics Engineering

Specialization – Power Electronics

Model Question Paper - I

Second Semester

MEEPE 201    SOLID STATE DC AND AC DRIVES

(Regular – 2011Admission onwards)

Time: Three hours                                  						Max. Marks: 100

(Answer all Questions)

- (a) Explain the motor performance parameters and input supply performance parameters of Single Phase DC Drives                                                                                         (  10 Marks )

(b) The speed of a10 hp, 230V, 1200 rpm separately excited dc motor is controlled by a single phase full converter. The rated motor armature current is 38A and the armature resistance is 0.3Ω.The ac supply voltage is 260V.The motor voltage constant is Ka Φ =0.182V/rpm. Assume that sufficient inductance is present in the armature circuit to make the motor current continuous and ripple free.

Rectifier Operation(Motoring Action) For a firing angle α =300 and rated motor current calculate

- The motor torque

- The speed of the motor

- The supply power factor

Inverter Operation(Regenerating Action)  The polarity of the motor back emf Eb is reversed by reversing the field excitation. Calculate

(i)The firing angle to keep the motor current at its rated value.

(ii)The power fed back to the supply                                                                        (15 Marks)

OR

2         (a) Explain the working of a single phase semi converter fed dc drive  with wave forms and basic equations                                                                                                                 (15 Marks)

(b) Explain the semi converter operation of full converters for power factor improvement

(10 Marks)

3         (a) Explain the working of Three Phase semi converter fed dc drive                     (10 Marks)

(b)A 80kW, 440V, 800 rpm dc motor is operating at 600 rpm and developing 75% rated torque is controlled by 3-Φ,six pulse thyristor converter.If the back emf at rated speed is410V, determine the triggering angle of the converter. The input to the converter is 3-Φ,415V,50Hz ac supply                                                                                         (15 Marks)

OR

4          (a) Explain the working of a Three Phase full converter fed  dc drive. Draw the waveforms for

α = 600and α = 900                                                                                                (10Marks)

(b) Explain  the non-circulating current and circulating current mode of operation of

Three phase Dual converters                                                                                                         (15Marks)

5          (a) What are the advantages of chopper fed drives                                                    (5 Marks)

(b) Explain the operation of a single quadrant chopper fed  separately excited

dc motor drive                                                                                                      (10 Marks)

(c) The speed of a separately excited dc motor is controlled by a chopper. The dc supply

Voltage is 120V, armature circuit resistance is 0.5Ω, armature circuit inductance is

20mH and motor constant is Ka Φ = 0.05V/rpm.The motor drives a constant torque load

Requiring an average armature current of 20A.Assume that  motor current is continuous.

Determine (a) The range of speed control (b) The range of duty cycle α                 (10 Marks)

OR

6          (a) What are the different chopper configurations?. Explain.                               ( 15 Marks)

(b) Explain the principle of multiphase choppers                                                    (10 Marks)

7         (a) Explain the rotor resistance control schemes for induction motors                    (10 Marks)

(b) Explain the different slip power recovery schemes for induction motor control (15 Marks)

OR

8       (a) Explain the stator voltage control schemes for speed control of induction motors(10 Marks).

(b) Explain the principle of vector control of induction motors                                   (15 Marks)