M.TECH. DEGREE EXAMINATION

Branch : Electrical and Electronics Engineering

Specialization – Power Electronics

Model Question Paper - I

Second Semester

MEEPE 204  MODELING AND ANALYSIS OF ELECTRICAL MACHINES.

(Regular – 2011Admission onwards)

Time: Three hours                                  					Max. Marks: 100

Answer all Questions.

I (a) 	Explain the function of magnetic coupling field in an electromechanical energy                                                              conversion device.                                                                             [10 marks]

(b) 	Derive the expression for stored magnetic field energy in a doubly-excited system. 								      	[15 Marks]

OR

(c)	 Derive the expressions for mechanical work done and electromagnetic force in the case of a singly excited system.                                                [25 Marks]

II	Derive an expression for the air-gap MMF in a 2-pole, 3-phase, Y-connected salient pole synchronous machine.                                                      [25 marks]

OR

Derive the voltage and torque equations of a d.c.machine.                  [25 Marks]

III  	Derive the induction machine model in arbitrarily rotating reference frame.

[25 Marks]

OR

Draw the arbitrary reference -frame equivalent circuits of a 3-phase, symmetrical induction machine using voltage equations in arbitrary reference –frame variables.

[25 Marks]

IV 	For a 2-pole, 3-phase, Y-connected, salient –pole synchronous machine, derive the voltage equations in machine variables and in arbitrary reference- frame variables.

[25 Marks]

OR

Draw the equivalent circuits of a 2-phase unsymmetrical induction machine using voltage equations in stationary reference-frame variables.

[25 Marks]