- Reg. No………………….

- Name……………………

- M.TECH DEGREE EXAMINATION

- Model Question Paper

Second semester

Branch: Electrical and Electronics Engineering;

Specialization: Power Systems

MEEPS 201  POWER SYSTEM CONTROL AND SECURITY

(2011 admission onwards)

[Regular/ Supplementary]

Time: Three hours						                   Maximum: 100 marks

Answer all questions

Each full question carries 25 marks

1.(a) Explain the fundamentals of Automatic generation control in isolated power systems

and  also in interconnected power systems                                                   (13 marks)

(b) A small system consists of 4 identical 500 MVA generating units feeding a total load of

1,020 MW. The inertia constant H of each unit is 5 on 500 MVA base. The load varies by

1.5% for a 1% change in frequency. When there is a sudden drop in load by 20 MW,

(i)Determine the system block diagram with constants H and D expressed on 2,000 MVA

base; (ii)Find the frequency deviation assuming that there is no speed governing action

(12 marks)

Or

2. (a) Discuss with a block schematic diagram  the implementation of AGC for one control area

(13 marks)

(b) A power system has a total load of 1,260 MW at 60 Hz. The load varies by 1.5% for every

1% change in frequency (D=1.5). Find the steady state frequency deviation when a 60 MW

load is suddenly tripped if (i) there is no speed control;(ii) the system has 240 MW of

spinning reserve evenly spread among 500 MW of generation capacity with 5% regulation

based on this capacity. All other generators are operating with valves wide open. Assume

that the effect of governor dead bands is such that only 80% of the governors respond to

the reduction in system load.                                                                                  (12 marks)

3. (a) Explain the use of shunt reactors and shunt capacitors for voltage control and their

application to transmission and distribution system                                               (13 marks)

(b) Explain the automatic voltage regulation of distribution systems                           (12 marks)

Or

4. (a) Explain operation of Thyristor controlled reactor(TCR) and Thyristor switched

capacitor(TSC).                                                                                                       (13 marks)

(b) Discuss the application of tap changing transformers to transmission systems     (12 marks)

5. (a) Explain maximum likelihood criterion with an example                                        (13 marks)

(b) Explain detection and identification of bad measurement                                      (12 marks)

Or

6. (a) Explain state estimation                                                                                          (12 marks)

(b) What are the various aspects of power system security                                          (13 marks)

7. (a) Explain SCADA                                                                                                      (13 marks)

(b) Explain contingency analysis                                                                                  (12 marks)

Or

8. (a) Explain principles of Load Shedding                                                                      (10 marks)

(b) Explain real time control of power system using a transition diagram during various modes

of operation                                                                                                             (15 marks)

[4 x 25 = 100 marks]