- Reg. No………………….

- Name……………………

- M.TECH DEGREE EXAMINATION

- Model Question Paper

Second semester

Branch: Electrical and Electronics Engineering;

Specialization: Power Systems

MEEPS 206 - 2   EHV AC AND DC TRANSMISSION (ELECTIVE IV)

(2011 admission onwards)

[Regular/ Supplementary]

Time: Three hours						                   Maximum: 100 marks

Answer all questions

Each full question carries 25 marks

- (a) Draw equivalent circuit of a long transmission line. Derive the relation between

sending and receiving end voltages and currents                                            (15 marks)

(b) A 3 phase overhead transmission line has a total series impedance per phase of

200 ∟800 ohms and a total shunt admittance of 0.0013∟900  seimens per phase. The

line delivers a load of 80 mW at 0.8 pf lagging and 220 kV between the lines.

Determine the sending end voltage and current by rigorous method.             (10 marks)

Or

- (a) With respect to long transmission line, give physical interpretation of the terms

‘characteristic impedance’ and ‘propogation constant’. What is meant by surge

impedance?                                                                                                     (10 marks)

(b)  A long transmission line has the following auxiliary constants: A=D=0.0945∟1.020,

B=82.3 ∟73.030 Ω and C=0.001376 ∟90.40  s. Determine (i) Equivalent T network;

and  (ii) π network                                                                                          (15 marks)

- (a)  Explain the effect of over voltages due to low induction current and capacitive

current                                                                                                            (15 marks)

(b)  Write short note on sequential impedance                                                      (10marks)

Or

- Write short note on following

- Voltage divider

- High speed oscilloscope

- Peak voltmeter and sphere gap

(iv)       Digital recorder

- Measurement of partial discharge

(5 x 5=25 marks)

- (a)What is the principle of HVDC system operation? Discuss the technical and economic

advantages of d.c. systems over a.c. systems                                                   (10 marks)

(b)With a block schematic, explain the main components of a HVDC transmission

system                                                                                                               (15 marks)

Or

- (a)Draw the equivalent circuit of a 3 phase full wave bridge converter and the waveforms

of voltages and currents. Obtain the expansion for average direct voltage of the

converter                                                                                                          (15 marks)

(b) Discuss briefly on the abnormal operations of converters                              (10 marks)

- (a)Explain the basic principles of HVDC link control. What are the factors that influence

the selection of control characteristics                                                             (15 marks)

(b) With a block schematic, explain the current control system with equidistant pulse

control                                                                                                            (10 marks)

Or

- (a) Discuss on the filters used on both sides of HVDC system to reduce the harmonics

(15 marks)

(b) Discuss briefly on the modeling of HVDC system                                         (10 marks)

[4x25=100 marks]