# Register No:…………………

Name          :…………............

M.TECH. DEGREE EXAMINATION

Model Question Paper - I

Second Semester

Branch: Information Technology

Specialization: Network Engineering

## MITNE 205.2  MODERN DIGITAL COMMUNICATION TECHNIQUES

(Elective III)

- (Regular – 2011 Admission onwards)

### Time: Three hours                                                               		    Maximum: 100 Marks

1 a) Explain the following terms:-

- Information and Information rate.

- Entropy

- Shannon’s theorem

- Channel capacity                                                                                               (10 marks)

b) Define and explain various multiplexing techniques used in communication systems.

(15 marks)

OR

2 a) Describe the scalar and vector communication over memory less channel.           (15 marks)

b) Explain :

- Shannon Hartley capacity theorem

- Shannon limit.                                                                                                    (10 marks)

3 a) Define code word, code rate and hamming weight. Also write short note on Hamming code.                                                                                                                               (15 marks)

b) Write a short note on:

- Viterbi algorithm

- Cyclic code.                                                                                                       (10 marks)

OR

4 a) The generator matrix of (6, 3) systematic block code is given by,

G=  {{1,0, 0,  0, 1, 1},{0,1,0,1,0,1},{0,0,1,1,1,0} }

- Find code vectors.

- Find the parity check matrix

- Find the error syndrome.                                                                                   (15 marks)

(b) With a suitable example explain the Hamming and cyclic codes.                         (10 marks)

P.T.O

5a) For a 16-ary PCM system if,

P1=P2=P3=P4=0.1

P5=P6=P7=P8=0.05

P9=P10=P11=P12=0.075

P13=P14=P15=P16=0.025

Calculate the entropy and information rate assume fm= 3 KHz.                               (15 marks)

b) Explain FDM and TDM.                                                                                         (10 marks)

OR

6 a) Explain the transmitter and receiver for the Adaptive delta modulation system.  (10 marks)

b) An AM signal appears across a 50Ω load has the following equation:-

V (t) =12(1+ sin 12.566 x 10³ t) sin 18.85 x 10³t Volts.

- Sketch the envelope of this signal in time domain.

- Calculate the modulation index, side band frequencies, total power and bandwidth.

(15 marks)

7. Write short notes on:-

- Various noise parameters.

- Mouse in communication system.

- Intersymbol interference.

- QAM.                                                                                                                 (25 marks)

OR

8a) What is line coding? Draw the waveforms if the sequence is transmitted using:-

- Unipolar Rz

- Polar Rz

- AMI

- Split phase Manchester.

- M ary where M=4

Assume the binary sequence 11010011.                                                                    (15 marks)

b) Explain the QAM Transmitter and Receiver.                                                         (10 marks)