B. TECH DEGREE EXAMINATION

Fifth Semester

Branch: Mechanical Engineering, Production Engineering

ME 010 503 Advanced Mechanics of Materials

Time: Three hours                                                                                               Maximum: 100 Marks

Part A

Answer all the questions.

Each question carries three marks.

- Define the term state of strain at a point.

- Explain the terms plain stress and plane strain.

- What are the stresses in thick cylinder under axisymmetric load? Discuss.

- State Maxwell reciprocal theorem.

- What is the significance of membrane analogy

(5X3=15 marks)

Part B

Answer all the questions.

Each question carries three marks.

- What is a stress function? How do you relate the polar components of stress with stress function?

- Explain the effect of small central hole in a rotating cylinder.

- Explain the method of determination of constants in Lamis equation for thick cylinders

- State the Castiglianos’s theorem

- Derive the expression for torsion in a prismatic bar of elliptical cross section

(5X5=25 marks)

Part C

Answer all the questions

- The state of stress  at a point is characterized by the following rectangular stress components

σx = 20,  σy=10,  σz= 5      τ xy=-10,   τ yz=-15,  τ zx=-20

Find the values of principal stress and their directions.

Or

- Derive the compatibility equations what are its significance.

- Derive the expression for radial stress distribution over the thickness of a spherical shell

Or

- An alloy steel cylinder has a 120mm internal diameter and 444mm outside diameter. If it is subjected to an internal pressure of 125MPa,(outside pressure =0)

  - Determine the radial and tangential stress distribution and plot them

  - Determine the maximum principal shear stress.

- Determine the ratio of numerical values of σmax and  σmin for a curved bar of circular cross section if radius of curvature of centroidal axis is 5cm and h = r2-r1 = 40 cm.

Or

- Derive the expression for the pure bending of a bar with rectangular cross section.

- The stress field for a beam of length 2l and depth 2c under the bending moment M is given by σx= -3M/2c3 y, σy = σz = τ xy= τ yz= τ zx= 0. Find the total strain energy density in the beam.

Or

- Find an expression in terms of σy , σz , τ xy for the strain energy V per unit thickness of a cylinder in plane strain.

- Determine the shear stress induced and the angle of twist per unit length of a hollow shaft of dimension 80mmX40mm and wall thickness 5mm when subjected to a torque of 1 kN m. G= 1.3x104MPa

80mm

40 mm

Or

Or

- Derive the expression for torsion of hollow circular shaft

(5X12=60 marks)