Reg. No………………..

Name………………….

B.TECH DEGREE EXAMINATION,

Fifth Semester

Branches: Mechanical Engineering/Automobile Engineering/Production Engineering

ME 010 506 THERMODYNAMICS (M, U)

Time: Three Hours							  	         Maximum: 100 Marks

Use of Steam tables and Psychometric chart are permitted

Part A

Answer all questions.

- Explain the control volume and control mass concept.

- Write down the steady flow energy equation applied to Compressor and Turbine.

- What is the principle of increase of entropy?

- What are the Maxwell relations and what is their necessity?

- State Dalton’s law of partial pressures.

[5 x 3 = 15 marks]

Part B

Answer all questions.

- Write short note on Compressibility factor and the Law of corresponding states.

- Explain the temperature scales used in SI and English system.

- State Clausius Statement of Second Law and explain Third law of thermodynamics.

- Show that internal energy of an ideal gas is a function of temperature only.

- With a neat sketch explain the Mollier diagram.

[5 x 5 = 25 marks]

Part C

Answer any one full question from each module.

11. (a)   What do you mean by thermodynamic equilibrium?

(b)	3 m3 of air has a pressure of 5 bar and a temperature of 127oC. After it has received 650 KJ of heat, at constant volume, find the final temperature and pressure. Also calculate change in entropy.    							                     			    (6 + 6 = 12 marks)

Or

- (a) 	Explain the concept of continuum.

(b)	Determine the specific volume of refrigerant-134a at 1 MPa and 50oC, using (i) the ideal-gas equation of state. (ii) the generalized compressibility chart. Compare the values obtained to the actual value of 0.021796 m3/kg and determine the error involved in each case.

(Take R = 0.0815 kPa.m3/kg.K, Pcr = 4.059 MPa, Tcr = 374.2 K).                               (6 + 6 = 12 marks)

- (a)   Write a note on Joule Thompson effect.

(c) 	A gas in a piston-cylinder assembly undergoes an expansion process for which the relationship between pressure and volume is given by pVn = constant	. The initial pressure is 0.3 MPa, the initial volume is 0.1 m3 and the final volume is 0.2 m3. Determine the work for the process in kJ if (i) n = 1.5, (ii) n = 1.0, (iii) n = 0.			         		                		    (6 + 6 = 12 marks)

Or

- (a)   What are the modes in which energy is stored in a system?

(c)	Write the steady flow energy equation for a single stream entering and a single stream leaving a   control volume and explain the various terms in it.                                                      (6 + 6 = 12 marks)

- (a)	State and derive the Clausius inequality principle.

(b)	Prove that entropy is a property of the system. 				                 (6 + 6 = 12 marks)

Or

- (a)   With a neat sketch explain Carnot cycle.

(b)	A Carnot engine absorbs 200 J of heat from a reservoir at the temperature of normal boiling point of   water and rejects heat to a reservoir at the temperature of the triple point of water. Find the heat rejected, the work done by the engine and the thermal efficiency.	                              (6 + 6 = 12 marks)

- Explain Clapeyron equation and obtain the Clausius-Clapeyron equation from it.	                (12 marks)

Or

- Write notes on Gibbs and Helmholtz function and derive the two Tds equations.	                (12 marks)

- What is a pure substance? Explain with the help of P-V, P-T and T-S diagrams.                            (12 marks)

Or

- A vessel of volume 0.04 m3 contains a mixture of saturated water and saturated steam at a temperature of 250oC. The mass of the liquid present is 9 kg. Find the,

- pressure

- mass

- specific volume

- enthalpy

- entropy

- internal energy				                              		     		   (12 marks)

[5 x 12 = 60 marks]