B.TECH DEGREE EXAMINATION NOVEMBER 2012

Fifth Semester

Branch: Electronics and Communication Engineering

EC 010 504  ELECTRIC DRIVES AND CONTROL

Time:Three Hours                                                                                      Maximum 100 Marks

Part A

Answer all questions

1. Derive the emf equation of a DC machine.

2. Explain the torque-slip characteristics of a three phase induction motor.

3. Compare power MOSFET and BJT.

4. What is the basic principle of PWM?

5. Discuss about chopper fed drives.                                                                      (5 × 3 = 15 marks)

Part B

Answer all questions

6. Explain the need for starter with a DC motor.

7. Draw and explain the equivalent circuit of a two winding transformer.

8. Discuss different specifications of an SCR.

9. Explain the role of a free wheeling diode in a single phase half controlled circuit with R-L load.

10. Explain two and four quadrant drives.               .                                               (5 × 5 = 25marks)

Part C

11. (a) Explain the different types of DC Generators and its applications.                           (5 marks)

(b) A dc shunt generator gave the following open-circuit characteristic when drven at 750 rpm.

Field Current (A) :       0.5              1              1.5                 2                2.5

Emf (V)                :        50              84             105               120             131

- If the machine is run as a shunt generator at 750 rpm, to what voltage will it excite with shunt field resistance equal to 80 Ω ?

- What is the critical value of shunt field resistance?

- What is the critical speed when the shunt field resistance is 80 Ω ?             (7 marks)

OR

12. (a) Draw the expression for torque developed in a DC motor.                                       (4 marks)

(b) Explain Swinburne’s test for determining the efficiency of a DC machine. State its

advantages and disadvantages.                                                                                   (8 marks)

13 .(a) In a 50 kVA transformer, the iron loss of 450 W and full-load copper loss of 850 W.

Find the efficiency at full-load and half full-load at 0.8 p.f. lagging.                        (7 marks)

(b) Explain the working of a Synchronous motor                                                           (5 marks)

OR

14.(a) Draw and explain the phasor diagram of a single phase transformer for resistive load.

(5 marks)

(b)What are the different classifications of single phase induction motors? Explain each.

(7 marks)

15.(a) Explain the two transistor analogy of an SCR.                                                           (6 marks)

(b) Explain the static characteristics of a MOSFET                                                        ( 6 marks)

OR

16.(a) With neat diagram explain the RC triggering circuit of an SCR.?                             (6 marks)

(b) What is an IGBT? Explain its characteristics.                                                            (6 marks)

17.(a) Explain the principle of phase control.                                                                       (4 marks)

(b)Explain the  operation of a single phase fully controlled bridge circuit with RLE load.

(8 marks)

OR

18.(a) Explain the characteristics and features of current source inverters. Compare VSI and CSI.

(8 marks)

(b) Draw the block diagram of UPS and explain its principle of operation.                    (4 marks)

19. Explain in detail speed control of 3 phase induction motors and give the principle.    (12 marks)

OR

20. With neat waveforms, explain the operation of a single phase fully controlled bridge rectifier

dc drive. Derive the performance parameters.                                                              (12 marks)