EASWARI ENGINEERING COLLEGE

# DEPARTMENT OF ECE

# QUESTION BANK

##### Subject Code/ Name 	:  EC 2403- RF and Microwave Engineering

Degree/Branch	 	 : B.E / ECE

### Year/Sem/Sec	 	 : IV / 7th / A & B

# UNIT III (MICROWAVE Passive COMPONENTS)

## PART A

- What are matched terminators? May/Jun 2014.

- What are ferrites? Why is it needed in circulators? May/Jun 2014.

- What are the composition of ferrite? Nov/Dec 2013.

- What is Gyrator? Nov/Dec 2013.

- Give the significance of Rat-race junctions. May/Jun 2013.

- Power at the input port is 900mw. If this power is incident on 20dB coupler with directivity 40dB, what is the coupled power and transmitted power. May/Jun 2013.

- Draw the diagram of H-plane Tee junction.				 Nov/Dec 2012.

- A directional coupler is having coupling factor of 20dB and directivity of 40dB. If the incident power is 100mW, what is the coupled power? 					Nov/Dec 2012

- State the unity property of S-matrix. 					May/Jun 2012

- What are the differences between Gyrator and phase changer? 		May/Jun 2012

- Draw a structure of two hole direction coupler.                                          Nov/Dec 2011

- Find the resonant frequency of the TE101 mode of an air filled rectangular cavity of Dimensions 5 cm 4 cm 2.5 cm.        						Nov/Dec 2011

- Define a reciprocal network.						Nov/Dec’04

- State the properties of scattering matrix.				         	Nov/Dec‘04,’06

- What should be the minimum radius of curvature of E-plane bend and H-plane bend? Validate your answer.							           		 Nov/Dec ‘04

- When can you say that a termination is matched to a system? Give the structure of any such termination and state the significance of that structure.                             Nov/Dec ‘04

- Define coupling factor and directivity of a directional coupler.	            Nov/Dec 2004

- Write down the scattering matrix of an ideal lossless transmission line of length ‘l’ units. May’05

- Find the magnitudes of the scattering parameters for a reciprocal two-port microwave component having the VSWR as 1.4 and insertion loss as 1dB.				Apr/May 2005

- A waveguide load is used to absorb 2W of average power. The reflected power is 3mW. Find the magnitude of VSWR.							Apr/May 2005

- State phase shifting property of S-parameters.				Nov/Dec’05,’06

- State the two parameters that describe a directional coupler and define them. Nov/Dec’05

- For a reciprocal two-port microwave component, the magnitude of scattering co-efficient is 0.091. Find its VSWR.								May/June 2006

- Obtain the coupling co-efficient in dB of directional coupler if the magnitude of its corresponding scattering parameter is 0.707						May/June 2006

- Give the ‘K’ band frequency range.					May/June 2007

- Give the S-matrix of H-plane Tee.						May/June 2007

- What are waveguide corners, bends and twists?				Nov/Dec 2007

- Give the ‘X’ band frequency range.					Nov/Dec 2007

- What are the advantages of S-parameters over ‘Z’ or ‘Y’ parameters?         Nov/Dec 2007

- What do you mean by symmetry of scattering matrix?			Apr/May 2008

- Define Directivity of a Directional coupler. 					Apr/May 2008

- What are the basic parameters to measure the performance of a directional coupler? Nov/Dec’08

- What do you mean by Faraday rotation isolator? 					Nov/Dec’08

- State the principle advantage of microwave frequencies over lower frequency. 	May/June ‘09

- Write down the S-parameters of a transmission line of length ‘l’ and phase constant ‘β’.

- Why S parameters are used at microwave frequencies?

- Write the S-matrix of an isolator.

- List few applications of microwave.

- Give the ‘Ku’ band frequency range.

- Define insertion loss, transmission loss, and reflection loss and return loss in terms of S-parameters.

UNIT III (MICROWAVE COMPONENTS)

PART-B

- Draw and explain the operation of Magic Tee. Explain its application in the construction of a 4 – port circulator.(8) May/Jun 2014

- Find the directivity in dB for a coupler if same power is applied in turn to input and output of the coupler with output terminated in each case in a matched impedance. The auxiliary output readings are 450 mW and 0.710 µW.(8) May/Jun 2014

- Draw and explain the various types of attenuators and phase shifters. What is a gyrator?

May/Jun 2014

- Describe the following with neat sketch Nov/Dec 2013

(i) Magic Tee   (8)     (ii) Directional coupler.   (8)

- Explain the operation of following microwave passive devices. Nov/Dec 2013

(i) Circulator   (8)    (ii) Isolator.  (8)

- Explain the properties of E-plane Tee? Derive the expression of scattering matrix for directional coupler. May/Jun 2013

- Explain the properties of magic Tee and derive scattering matrix for it. May/Jun 2013

- State and explain the properties of S-parameters. Derive the S- parameters of a Directional Coupler.

- (a) Explain how directional coupler can be used to measure reflected power.(8)

(b) Explain the properties of H-plane Tee and give reasons why it is called shunt Tee. (8)

Nov/Dec 2012

- (a) Derive the equation for the scattering matrix of magic Tee(8)

(b) Differentiate between the circulators and isolators.(8) 		Nov/Dec 2012

- (a)Explain the concept of Two hole directional coupler and derive its S-matrix (8)

(b) Draw and explain the operation of an Isolator. (8) 			May/Jun 2012

- (a) Explain the construction of Magic Tee and derive its S-matrix. How can a isolator be designed using 3 port circulator? (8)

(b) Describe the principle of Microwave transmission through ferrite. Explain how a gyrator is designed based on this effect.(8) 					May/Jun 2012

- A four port network has the scattering matrix shown below

0.1∟900    0.8∟-450     0.3∟450           0

0.8∟-450           0                0         0.4∟450

0.3∟-450           0                0          0.6∟-450

0.4∟450       0.6∟-450          0

S=

(i) Is this network lossless?

(ii) Is this network Reciprocal?

(iii) What is the return loss at port 1 when all other ports are

terminated with matched loads?

(iv) What is the insertion loss and phase dialog between ports 2 and 4, when all other ports are terminated with matched loads?

(v) What is the reflection coefficient seen at port 1 in a short circuit is placed at the terminal plane of port 3, and all other ports are terminated with matched loads?   (16) Nov/Dec 2011

- Discuss the structure and principle of operation of : (i) Isolator (8) (ii) Circulator.  (8) Nov/Dec 2011

- Prove that it is not possible to construct a perfectly matched, lossless, reciprocal three-port junction.									Nov/Dec 2004

- Explain the working of Faraday rotation isolator.				Nov/Dec 2004

- Explain how power splitting occurs in i) E-plane Tee ii) H plane Tee iii) magic Tee.

Nov/Dec 2004

- State and prove the properties of scattering matrix. 	Apr/May 2005, May/June 06 & 09

- In a two hole directional coupler, prove that the power coming out in the coupled port is finite and that in the fourth port is zero.						Apr/May 2005

- Find the scattering matrix of an ideal, symmetrical, lossless magic T-junction.

Apr/May 2005

- From the first principle, derive the scattering matrix of an ideal directional coupler. Hence or otherwise write down S matrix of Hybrid coupler and Magic Tee. May’07,Nov/Dec ’05,’07

- What is the value of directivity of an ideal coupler? Why?			Nov/Dec ’05,’07

- An isolator with characteristic impedance of 50 ohms has forward and reverse loss of 0.9 dB and 32 dB respectively. Its input and output VSWRs are 1.09 and 1.16 respectively when a 50 ohms load terminates it. Determine its S-parameters. Assume zero phase components.	Nov/Dec’05

- Discuss the following properties of S matrix:

i) Symmetry property ii) Unitary property iii) Phase shift property. 			May/June ‘07

- What is a scattering matrix? Derive scattering matrix formulation for N port network. Nov’06, May’07

- Derive S-matrix for Magic Tee. 					Nov/Dec’08, May/June ‘07

- Explain with diagrams waveguide corners, bends and twists.		May/June 2007

- What is an isolator? Write down its S-parameters.				May/June 2007

- What is a hybrid ring? With the help of a neat diagram explain its working principle.

Nov/Dec 2007

- Derive S-matrix of E plane Tee using S-parameter theory.			Nov/Dec 2007

- What is scattering matrix? Derive scattering matrix formulation for n-port nework.-8M Apr/May’08

- Derive the S-matrix representation of multiport network.-8M 		Nov/Dec 2008

- Derive scattering matrix for E-plane Tee using ‘S’ parameter theory.-8M 	Apr/May 2008

- What is the function of an isolator? Write down its ‘S’ parameters-4M 	Apr/May 2008

- Explain the working of a circulator and hence obtain the scattering matrix of the circulator from the first principle.-12M 							Apr/May 2008

- Prove that it is impossible to construct a perfectly matched, lossless, reciprocal there port junction.- 8M 								Nov/Dec 2008

- Derive the S-matrix of a directional coupler. –8M 				Nov/Dec 2008

- List out the characteristics of the Magic-T when all the ports are terminated with matched load.-8M

- How can you synthesize a rat-race hybrid coupler from magic-tee? –8M	Nov/Dec 2008

- Prove that the impedance and admittance and admittance matrices are symmetrical for a reciprocal junction.-8M 							Nov/Dec 2008

- (i)For a two-port network with mismatched load,derive the equation for input reflection coefficient.

(ii) Deduce S-parameters in terms of ABCD parameters. 			Nov/Dec 2008

UNIT V - MICROWAVE TUBES AND MEASUREMENTS

# PART A

- What are the errors in impedance measurement? May/Jun 2014

- Define: Convection current of TWT. May/Jun 2014

- Distinguish between TWT and Klystron. Nov/Dec 2013

- Define SWR. Nov/Dec 2013

- What is Gunn Effect? Name the materials that exhibit Gunn effect? May/Jun 2013

- Draw the voltage wave forms of a TRAPATT diode. May/Jun 2013

- Compare O-type tube and M-type tube.				 Nov/Dec 2012

- What are the errors possible in standing wave ratio measurements Nov/Dec 2012

- Enumerate the significance of slow wave structures in TWTs. 	May/Jun 2012

- List a few sensors used for Microwave power measurement.	 May/Jun 2012

- State any four high frequency limitations. 				Nov/Dec 2011

- A helix travelling wave tube operates at 4GHZ, under a beam voltage of 10kv and beam current of 500mA. If the helix is 25and interaction length is 20 cm, find the gain parameter.

Nov/Dec 2011

- What are slow wave structures? Why and where they are used? 		Nov’04, May’07

- Can a two-cavity Klystron amplifier be used as an oscillator? If yes, how? 	Nov/Dec’04

- Draw four types of slow wave structures in TWT. 			Apr/May’05, May/June‘06

- State any two differences between TWT and Klystron amplifier.  Apr’05, May & Nov’06, May’07

- Draw the electrical equivalent of a cavity type Magnetron. 		Apr/May 2005, June 2006

- What is C ring tuning with respect to magnetron?				May/June 2006

- What is strapping with respect to magnetron?				May/June 2006

- What are pushing and pulling in magnetron?				May/June 2006

- Why magnetron is called as crossed field device?				May/June 2007

- What is meant by velocity modulation?					May/June 2007

- How are the oscillations that are spontaneously generated in a TWT suppressed?May/June’07

- What is the condition for oscillation in Reflex klystron?			Nov/Dec’07

- What is velocity modulation? 						Apr/May’08

- Define beam loading. 							Nov/Dec’08

- What is meant by Hull cut-off voltage? 					Nov/Dec’08

- Why the conventional tubes like triode, tetrode cannot generate microwave power? May/June’09

- Name some types of slow wave structures. 				May/June ‘09

- For what measurements would you use a slotted line carriage? What should be the minimum length of the slot in it?							Nov/Dec 2004

- What are the measurements that can be performed using (a) Spectrum Analyzer (b) Network Analyzer?								Nov/Dec 2004

- Compare a barreter with a thermistor.					Apr/May 2005

- What are the minimum and maximum values of VSWR? Give the corresponding values of reflection coefficients.							Apr/May 2005

- What are the three quantities to be measured to determine the impedance of a load at microwave frequencies?							Nov/Dec 2005

- Why is the slot located at the center of the slotted section?		Nov/Dec 2005

- What are low VSWR and high VSWR and name the method followed to measure high VSWR.										Nov/Dec 2005

- What is the difference between barretter and thermistor?		May/June 2006

- Name the sources of error in measurement.				Nov/Dec 2006

- What is a bolometer? Give two examples.				May/June 2007

- Name two methods to measure impedance.				Nov/Dec 2007

- Define return loss and write its expression. 				Nov/Dec 2007

- What is the principle by which high power measurements could be done by calorimetric method?

Apr/May 2008

- Write the main demerits of single bridge power meter. 			Nov/Dec 2008

- What does the accuracy of phase measurement depend on while measuring the impedance using reflectometer method? 							Nov/Dec 2008

- Draw a setup to measure the frequency.					May/June 2009

- Mention two methods to measure microwave power. 			May/June 2009

- What is the use of attenuator in TWT?

- What is Hull cut off condition? Give an expression for it.

- Draw the schematic diagram of a two-cavity klystron.

- What is bunching in klystron?

- What is multi-cavity klystron?

- What are the advantages of multi-cavity klystron?

UNIT V - MICROWAVE TUBES AND MEASUREMENTS

# PART B

- Explain the launching process of a two – cavity klystron and derive expression for its optimum bunching distance Lopt. May/Jun 2014

- Describe the measurement of power at microwave frequencies in detail.(8) May/Jun 2014

- Explain the procedure to measure the impedance of a load.(8) May/Jun 2014

- Explain the working principle of Reflex Klystron and derive the expression of bunching parameter. Nov/Dec 2013

- (i) Write a detailed note on cylindrical magnetron (8) Nov/Dec 2013

(ii) Explain the procedure for measuring impedance at microwave  frequency with the aid of slotted line.(8) Nov/Dec 2013

- (i)  Describe how the frequency of a given source is measured. (8) May/Jun 2013

(ii) Explain the measurement of high VSWR with the help of block diagram.(8)

- Explain the principle of operation of the cavity klystron with neat sketch. A 250kW pulsed cylindrical magnetron has the following parameters. Anode voltage = 25 KV, Peak anode current = 25A, magnetic field B = 0.35Wb/m2, Radius of cathode = 4CM, Radius of cylinder = 8CM. May/Jun 2013

- Calculate efficiency of the magnetron, cyclotron frequency, cut-off magnetic field.

- (a)  Explain the operation mechanism of two-cavity Klystron amplifier with neat sketch. (8)

(b) A two cavity Klystron has the following parameters. V0 =1000V, R0= 40k ohms, I0=25mA, f= 3 GHz.Gap spacing in either cavity (d) = 1mm, Spacing between two cavities L=4 cm, Effective shunt impedance Rth = 30 k ohm. Calculate input gap voltage, voltage gain and efficiency. (8)

Nov/Dec 2012

- (a) Describe how the frequency of a given microwave source can be measured. (8)

(b) Explain how low VSWR can be measured using a microwave bench (8) 	Nov/Dec 2012

- (a) Draw the cross sectional view of Magnetron tube and explain how bunching occurs in it. Derive the expression for Hull cut-off voltage.   (12)

(b) What are practical applications of magnetron? Explain any one in detail. (4) May/Jun 2012

- Write notes on the following :

(a) Measurement of power (8)

(b) Measurement of impedance (8). 					May/Jun 2012

- Explain the operation of two cavity Klystron amplifier and compare it with traveling wave tube.

Nov/Dec 2011

- Explain the principle of microwave power measurements. (8)  (ii) Discuss slotted line method of VSWR measurement. (8) 							Nov/Dec 2011

- Draw a neat sketch showing a helix type TWT and describe he mechanism of electron bunching.											May/June 2007

- Draw the schematic diagram and explain the operation of two-cavity klystron. Derive expression for beam current and efficiency.						May/June 2007

- Describe with a neat sketch the constructional details and principle of operation of a Reflex klystron. With the help of Applegate diagram illustrate he phenomenon of bunching. Derive expression for beam current and efficiency.				Nov/Dec 2007

- Draw a neat sketch showing the constructional feaures of a cavity magnetron and explain why Magnetron is called as crossed field device-4M 				Apr/May ‘08

- With the applegate diagram, describe the mechanism of ooperatiion of two cavity klystron amplifier. Wrie the assumptions on which the analysis for RF amplification by this amplifier is based.-8M 								Nov/Dec 2008

- How can you analyse a TWTA circuit that uses a helix slow-wave non-resonant microwave guiding structure? –8M 								Nov/Dec 2008

- Explain the oscillation mechanism and the electron trajectory concept of magnetron oscillator. –8M

- Explain the working principle of two cavity klystron amplifier.-8M 		Nov/Dec 2008

- Explain the amplification process in TWT.-8M 				Nov/Dec 2008

- Write the assumptions on which the analysis for RF amplification by a two cavity klystron amplifier is based.-6M 								Nov/Dec 2008

- Explain with a neat figure, how TWT is used as an microwave oscillator?- 10M May/June’09

- List and explain various applications of TWT.-6M 				May/June 2009

- How bunching takes place in klystron amplifier? Derive an expression for optimum distance of klystron in bunching process.

- Discuss the classification of bolometers.					Nov/Dec 2004

- Explain the double minima method VSWR measurement.			Nov/Dec 2004

- Draw the experimental set up to find the high VSWR and mention the procedure. Apr/May‘05

- Explain the method of measuring impedance at microwave frequencies. 	Nov/Dec 2005

- Explain impedance measurement by reflectometer.			May/June 2007

- With neat block diagram explain how a network analyzer can be used o measure he amplitude and phase of a signal over a wide frequency range. Also explain the measurement of reflection and transmission parameters and ‘S’ parameters. 				May/June 2007

- Describe in detail with block diagram the measurement of VSWR through return loss measurement.											Nov/Dec 2007

- Explain in detail the measurement of load impedance through slotted line method.Nov/Dec’07

- With neat block diagram explain the Insertion loss and Attenuation measurements. Apr/May’08

- Explain the measurements of load Impedance by slotted line method. 	Apr/May ‘08

- Explain the measurement of cavity ‘Q’ by slotted line method.		 Apr/May ‘08

- What are the methods available for power measurement in Microwave circuits? Explain.Nov’08

- Explain the working principle of Network Analyser. –8M 			Nov/Dec 2008

- With block schematic explain the reflectometer method to measure cavity Q.-8M    Nov/Dec’08

- Explain the basic priniciple of operation of a bolometer. –8M		 May/June 2009Z

- Describe a technique of measuring the phase shift provided by a network.- 8M May/June’09

- Explain with a block diagram how frequency of an unknown microwave signal can be measured.

May/June 2009

UNIT IV -	MICROWAVE SEMICONDUCTOR DEVICES

# PART A

- What are the factors that reduce the efficiency of IMPATT diode? May/Jun 2014

- What is negative resistance in Gunn diode? May/Jun 2014

- What is a step recovery diode. Nov/Dec 2013

- Mention the ideal characteristics of dielectric material in MMIC. Nov/Dec 2013

- What is Gunn effect? Name the materials that exhibit Gunn effect? May/Jun 2013

- Draw the voltage wave forms of a TRAPATT diode. May/Jun 2013

- Name the advantages of parametric amplifiers. 				Nov/Dec 2012

- State the transferred electron effect. 					Nov/Dec 2012

- Define Transconductance and output resistance of a MESFET. 		May/Jun 2012

- Can Inductive elements be fabricated in MMICs? Justify your answer. 	May/Jun 2012

- Compare tunnel diode and normal P – N diode.               			Nov/Dec 2011

- List the advantages of parametric amplifier. 				Nov/Dec 2011

- Define Gunn effect. OR State Gunn effect.		May/June’09, Apr/May ’08, Nov/Dec ‘04

- What is the necessary condition for an IMPATT diode to produce oscillations?  Nov ‘04

- State any two differences between microwave transistors and transferred electron devices.							   				Apr/May ‘05, Nov/Dec ‘07

- Which two-phase angles make the voltage and current in Avalanche transit time devices to be out of phase by 180˚?							Nov/Dec 2005

- Mention the advantages and limitations of parametric amplifier. 		May’06, Nov‘06

- What is transferred electron effect?					May/June ‘07

- List two applications of parametric amplifier.				May/June ‘07

- What are HEMT’s?							Nov/Dec ‘07

- What is MESFET?								Nov/Dec ‘07

- What are avalanche transit time devices?					Nov/Dec ‘07

- Mention any two applications of IMPATT diode.				Nov/Dec’07

- State the transferred electron effect. 					Nov/Dec’08

- What are the major disadvantages of IMPATT diodes? 			Nov/Dec’08

- What is Gunn effect? What do the acronyms IMPAT, TRAPATT and BARIT stand for? May‘09

- Mention the basic characteristics required for microwave integrated circuit. Nov ‘07

- List the various types of strip lines used in MMIC.				Nov/Dec ‘07

- List the advantages of Microwave IC’s. OR Write the advantages of MIC’s	May ’08 & ‘09

- List out the advantages of Monolithic Microwave Integrated Circuits. 	Nov/Dec ‘08

- Mention the applications of MMIC

- Mention the fabrication techniques for MMIC

UNIT IV -	MICROWAVE SEMICONDUCTOR DEVICES

# PART B

- Explain the operating principle of a Gunn diode. Describe its domain formation and various modes of operations. (8) May/Jun 2014

- Explain the operating principles of varactor and step recovery diodes (8) May/Jun 2014

- What are the materials used for MMIC fabrication? Explain with neat diagrams the fabrication process of MMICs. May/Jun 2014

- Explain the principle of operation of Tunnel diode and TRAPATT device. Nov/Dec 2013

- Describe the Gunn effect with the aid of Two-valley model theory.  (8) Nov/Dec 2013

- Draw the physical structure and doping profile of IMPATT diode and explain in detail.

(8) Nov/Dec 2013

- With the aid of suitable sketch discuss construction, materials characteristics and working of IMPATT diode. May/Jun 2013

- With the help of two-valley theory, explain how negative resistance is created in Gunn diodes.

May/Jun 2013

- (a) Explain the tunneling action in a tunnel diode. (8)

(b) Draw the construction and explain the working of IMPATT diode. (8) Nov/Dec 2012

- With the help of two-valley, explain how negative resistance can be created in Gunn diode. Mention its applications. 							Nov/Dec 2012

- (a) With energy band structures, explain the tunneling phenomenon in Tunnel diodes. (8)

(b) Explain the design and operation of parametric amplifiers. (8)		 May/Jun 2012

- (a) Explain any four modes of operation of a Gunn diode. (8)

(b) An IMPATT diode has a drift length of 2 µm. Determine the drift time of the carriers and the operating frequency of the IMPATT diode. (8) 				May/Jun 2012

- With neat diagram, Explain the working principle of Gunndiode. Mention its application. (16)

Nov/Dec 2011

- Explain the characteristics and working of : (i) Avalanche transit time diode. (8)

(ii) Parametric amplifier. (8)  						Nov/Dec 2011

- With the help of energy-band diagrams explain the working of a tunnel diode. Nov’04

- Derive Manley-Rowe power relations. Use the above relations to find the power gain of an up-converter and down converter. 				Nov/Dec ‘04& ‘05, May/June’07

- Using RWH theory, explain two-valley model of GaAs. Hence or otherwise show how negative resistance characteristics is obtained in a Gunn diode. 			Nov/Dec ’07, Apr ‘05

- Write in detail the principle mechanism of operation and the application of IMPATT device.Dec’05

- Give the voltage-current waveform with respect to time of a TRAPATT diode and write down its characteristics.								Nov/Dec 2006

- Write short notes on

- Gunn diode (ii) IMPATT device					May/June 2007

- Explain the constructional details and principle of operation of GaAs MESFET with neat diagram and characteristic curves.							Nov/Dec 2007

- With neat diagram explain the constructional details of an IMPATT diode and explain the principle.											Nov/Dec 2007

- Explain various modes of operation of Gunn diode.		 May/June’09, Nov/Dec’07

- Enumerate with appropriate equations the power frequency limitations of BJT’s at high frequencies. 								Apr/May 2008

- Write short notes on: (i) Microwave FET’s –8M (ii) HEMT – 8M 		Apr/May 2008

- Draw the gepmetrical diagrams of GaAsFET and explain.-8M		 Nov/Dec 2008

- What are the modes of operation that result in microwave oscillations in a Gunn diode?      Explain.-8M 								Nov/Dec 2008

- Describe the principle of operation, the cases of input resistance at signal frequency and the circuit performance of parametric amplifier.-8M					 Nov/Dec 2008

- Explain the construction and DC operating of IMPATT diode.-8M 		Nov/Dec 2008

- Draw the microwave equivalent circuit of a bipolar transistor –8M 		May/June 2009

- Explain the operation of a tunnel diode.-8M 				May/June 2009

- Discuss the differences between transferred electron devices and avalanche transit time devices.8M 								May/June 2009

- Explain in detail with suitable diagram the fabrication techniques of a MMIC circuit.Nov/Dec’07

- Explain in detail with suitable diagrams the fabrication techniques of a monolithic microwave integrated circuit. 							Apr/May ‘08

- List the basic characteristics required for an ideal substrate maerial-4M 	Apr/May ‘08

- Write the advantages and limitations of parametric amplifiers. –6M 	Nov/Dec 2008

UNIT I - TWO PORT RF NETWORKS-CIRCUIT REPRESENTATION

# PART A

- List any four reasons for the wide use of RF. May/Jun 2014

- Give the relationship between [s] and [z]. May/Jun 2014

- Draw the equivalent circuit of an inductor at radio frequency. Nov/Dec 2013

- What is ESR? Nov/Dec 2013

- Define reciprocal and symmetrical networks May/Jun 2013

- Express power input and power output under matched conditions for a two-port network in terms of wave components. May/Jun 2013

- Name the properties fo S-parameters. 				Nov/Dec 2012

- Draw the equivalent circuit of practical capacitor. 			Nov/Dec 2012

- What are the advantages of Scattering parameters? 		May/Jun 2012

- Which component is represented by the scattering matrix 	 [ 0 1

1  0 ] May/Jun 2012

- Mention the limitations in measuring Z, Y and ABCD parameters at Microwave frequencies.

Nov/Dec 2011

- A 5dB attenuator is specified as having VSWR of 1.2. Assuming that the device is reciprocal, find the S parameters. 							Nov/Dec 2011

- What are the applications of RF?

- What is skin depth?

- Determine the radius of the AWG 26 wire?

- List the type of resistors?

- Draw the equivalent circuit of the high-frequency resistor?

- Draw the equivalent circuit of the high-frequency wire-wound resistor?

- What are the applications of High-frequency capacitors?

- Define ESR?

- Draw the frequency response of the impedance of an RFC.

- Draw the conventional diagram of a two-port network

- Draw the two-port network and multiport network

- Write the impedance matrix and admittance matrix of a 2-port network.

- Write the hybrid matrix and chain matrix of a 2-port network.

- Give the hybrid representation of input impedance & voltage feedback ratio

- Which parameters are used to analyze the cascaded networks?

- What is the ABCD matrix representation of a transmission line section with the length “L”?

- Give the matrix representation of a Pi-network?

- Give the matrix representation of a T-network?

- Write the expression for ∆h and ∆ABCD.

- How to find forward and reverse power gain of a 2-port network?

- State the properties of scattering matrix.

- Write down the scattering matrix of an ideal lossless transmission line of length ‘l’ units.

- What are the advantages of S-parameters over ‘Z’ or ‘Y’ parameters?

UNIT I - TWO PORT RF NETWORKS-CIRCUIT REPRESENTATION

# PART B

- With the help of S matrix concept prove the following properties. May/Jun 2014

- Symmetry

- Unity

- Zero and

- Phase shift.

- When do you prefer transmission matrix? Obtain the ABCD matrix of a transformer with turns ration of N:1.	(8) May/Jun 2014

- The impedance matrix of a certain lumped element network is given by Z11=4, Z12=2, Z21=2, Z22=4. Determine the equivalent scattering parameter matrix. (8) May/Jun 2014

- (i) Write a detailed note on ABCD parameters. (8) Nov/Dec 2013

(ii) The input of an amplifier has a VSWR of 2 and the output has a VSWR of 3. Find the magnitude of the S-Parameters S11 and S22 under matched condition. (8) Nov/Dec 2013

- A four port network has the scattering matrix shown below.

S11=0.1∟900, S12=0.6∟-450, S13=0.6∟450, S14=0, S21=0.6∟-450, S22=0, S23=0, S24=0.6∟450, S31=0.6∟450, S32=0, S33=0, S34=0.6∟-450, S41=0, S42=0.6∟450,  S43=0.6∟-450, S44=0.

- Is this network lossless?

- Is the network reciprocal?

- What is the return loss at port 1 when all others ports are matched?

Justify your answer.   (10)     Nov/Dec 2013

- Find the Z parameters Z11 and Z22 of the two port T-network. Nov/Dec 2013

- Formulate S-matrix for n-port network compute ABCD for a T-network. May/Jun 2013

- (a) Formulate scattering matrix for a n-port microwave network, (8)

(b) Give the [ABCD] matrix for a two-port network and derive its [S] matrix(8) Nov/Dec 2012

- The S-parameters of a two-port network are given by  S11 = 0.2∟900  S22= 0.2∟900   S12=0.5∟900   , S21= 0.5∟00

(a)Determine whether the network is lossy or not.(8) Nov/Dec 2012

(b) Is the network symmetrical and reciprocal? Find the insertion loss of network. (8)

- (a) Explain the properties of S-matrix and hence S-matrix representation of n-port network (8)

(b) if the impedance matrix of a simple device is [ 4  2

2   4], find its S-matrix  May/Jun 2012

- (a) Derive the S-matrices of linear loseless Microwave devices. (8)

(b) What are transmission matrices? Explain them and obtain their relationship with S-matrix. (8) 									May/Jun 2012

- A shunt impedance Z is connected across a transmission line with characteristic impedance Z0. Find the S matrix of the junction. (8)

List and explain the properties of S parameters. (8)                             Nov/Dec 2011

- The S parameters of a two part network are given by S11 =0.2∟00 , S22 =0.1∟00 , S12 =0.6∟900, S21 =0.6∟900.(i) Prove that the network is reciprocal but not lossless. (ii) Find the return loss at port 1 when port 2 is short circuited.  (16)                              Nov/Dec 2011

- Discuss in detail about RF behavior of passive components.

- Discuss in detail about the passive chip elements in terms of their sizes and electric characteristics.

- How to convert (a) ABCD to Y-parameters (b) ABCD to h-parameters?

- How to convert (a) h-parameters to ABCD parameters (b) ABCD parameters to Z-parameters?

- How to convert the Low frequency parameters to High frequency parameters?

- Briefly discuss about the applications of RF.

- State and prove the properties of scattering matrix.

- What is a scattering matrix? Derive scattering matrix formulation for N port network.

- Deduce S-parameters in terms of ABCD parameters.

UNIT II - RF TRANSISTOR AMPLIFIER DESIGN AND MATCHING NETWORKS

# PART A

- Define stability. May/Jun 2014

- What are the needs for impedance matching networks? May/Jun 2014

- Define transducer power gain. Nov/Dec 2013

- Give the expression that relates nodal quality factor (Qn) with loaded quality factor (QL).

Nov/Dec 2013

- Why impedance matching is required. What are other constrains required. May/Jun 2013

- Draw typical output stability circle and input stability circle. May/Jun 2013

- What are the considerations in selecting a matching network? 	Nov/Dec 2012

- Define power gain of amplifier in terms of S-parameters and reflection coefficients.

Nov/Dec 2012

- Distinguish between conditional and unconditional stabilities of Amplifier. May/Jun 2012

- A GaAs MESFET has the following parameters:  S11 = 0.65∟-1540, S12 = 0.02∟400, S21=2.04∟1850 and S22=0.55∟-300. Calculate its maximum stable power gain.													May/Jun 2012

- Write the expression for noise figure of a two port amplifier.		Nov/Dec 2011

- What is the need for impedance matching networks? 			Nov/Dec 2011

- Draw the generic amplifier system

- List the key amplifier parameters

- Define transducer power gain?

- What is unilateral power gain?

- Write the output stability circle equation.

- Write the input stability circle equation.

- What is the circle radius and location of center of output stability circle equation?

- Draw the output stability circles

- Draw the input stability circles

- When an unconditionally stability can be achieved?

- Write the stability factor expression.

- How the unstable BJT/FET can be stabilized?

- What is the equation for constant gain circle?

- What are the observations can be made from constant gain circle?

- What is unilateral figure of merit?

- Write the noise figure of a 2-port amplifier?

- List the noise parameters.

- Write the noise figure circle equation

- What are the observations can be made from noise figure circle equation?

- Why matching networks are needed?

- Draw the L-section network.

- Write the procedure for optimal power transfer.

- What are forbidden regions?

- What are the factors to be considered while selecting an L-section?

- Draw the general topology of a T-type matching network.

UNIT II - RF TRANSISTOR AMPLIFIER DESIGN AND MATCHING NETWORKS

# PART B

- Design a matching network to match a ZL = (10+j10)Ω to a 50Ω line. Specify the values of L and C at frequency of 1GHZ. (use smith chart). May/Jun 2014

- Derive the expressions for the following of a micro stripline matching network

(a) w/h ratio (b) Total Q factor (QT)  May/Jun 2014

- A MESFET operated at 5.7 GHz has the following S-parameters :

S11 = 0.5 ∟-600 ,  S12 = 0.02∟00

S21 = 6.5∟1150  , S22 = 0.6∟-350

Verify the circuit, whether it is unconditionally stable or not?  (6)     Nov/Dec 2013

- Write brief note on: Nov/Dec 2013

- Operating power gain (3)

- Available power gain (3)

- Noise figure. (4)

- Discuss the design procedure for T and π matching networks. Nov/Dec 2013

- Derive the transducer power gain for a transistor amplifier. Design LC network to match source impedance Zs = (50+j25) Ω to the load ZL = (25-j50) Ω. Assume ZO = 50 Ω, f= 2GHZ. Use smith chart. May/Jun 2013

- Discuss the smith chart approach to design the L-section and T-section matching networks.

May/Jun 2013

- A microwave amplifier is characterized by its S-parameters. Derive equations for power gain, available gain and transducer gain 						Nov/Dec 2012

- (a) What is a matching network? Why is this required? (8) 			Nov/Dec 2012

(b) design a lumped element  ‘LC’ network for matching ZL = 10+j10 ohm to a 50 ohm transmission line at 1 Ghz. (8)

- The S-parameters for a transistor is given below. Determine its stability and draw the input and output stability circles(Use Smith Chart).

S11 = 0.385∟-530, S12 = 0.045∟900, S21=2.7∟780 and S22=0.89∟-26.50. May/Jun 2012

- (a) Describe the process of visualizing the noise performance of a transistor by plotting noise circles on the S plane. (8)

(b) Explain microstripline matching networks. (8) 				May/Jun 2012

- Explain the following : (a) Impedance matching networks. (b) Microshipline matching networks. (16)  										Nov/Dec 2011

- A microwave transistor has the following S parameters at 10 GHZ, with a 50  reference impedance, S11 = 0.45∟1500 , S12 = 0.01∟-100 , S21=2.05∟100 , S22=0.4∟-1500 . The source impedance is 20ohms and load impedance is 30ohms. Compute the power gain, available gain and the transducer power gain.  (16)                               			Nov/Dec 2011

- Discuss in detail about various amplifier power relations. (479)

- Derive the rollet factor (490) ?

- From the first principle, derive the constant gain circle equations.

- Discuss in detail about the noise figure circles.

- Discuss in detail about microstrip line matching networks.

- An RF amplifier has the following values:

- S11 = 0.3Լ 00, S21 = 3.5L 850, S12 = 0.2L-100, S22 = 0.4L-450, Vs = 5V, Zs = 40 Ω, ZL = 73 Ω,   Z0= 50 Ω.

Find the following quantities

- Transducer gain, Unilateral transducer gain, available gain, operating power gain

- Power delivered to the load, available power, incident power to the amplifier.

- The output impedance of a transmitter operating at a frequency of 2Ghz is ZT=(150+j75) Ω. Using analytical approach, design an L-section matching network, such that the maximum power is delivered to the antenna whose input impedance is ZA=(75+j15) Ω.

- The output impedance of a transmitter operating at a frequency of 2Ghz is ZT=(150+j75) Ω. Using graphical approach, design an L-section matching network, such that the maximum power is delivered to the antenna whose input impedance is ZA=(75+j15) Ω.

- Design a T-type matching network that transforms a load impedance ZL= (60-j30) Ω into a      Zin= (10+j20) Ω input impedance and that has a maximum nodal quality factor of 3. Compute the values for the matching network components, assuming that matching network is required at f=1 Ghz.

Prepared By:							Approved By:

[A.PONRAJ] 								HOD/ECE