Easwari Engineering College

Department of Civil Engineering

Question Bank

ST 7006 Design of Bridges

Unit –I

Part- A

| 1. | How will you effective width of slab for a single concentrated load? |
| --- | --- |
| 2. | What are the forces acting on bridges? |
| 3. | How will you find reaction factor for guider using Courbon’s Theory? |
| 4. | Classify the types of bridges. |
| 5. | Explain the IRC Specifications for live load for IRC class AA loading? |
| 6. | Explain the loading standards of bridges |
| 7. | Explain the IRC Specifications for live load for IRC class A loading? |
| 8. | What are the span range for solid and Tee beam bridges? |
| 9. | Explain the IRC Specifications for live load for IRC class B loading? |
| 10. | Draw the loading standard for IRC class AA loading. |
| 11. | Explain the IRC Specifications for live load for IRC class 70R loading? |
| 12. | State the various loads and forces to be considered in the design of bridge members. |
| 13. | Explain the procedure for shear stress calculation in Solid slab bridges. |

Part-B

| 1. | Design a deck slab bridge for the following data: Clear distance between abutments : 7m Road : NH (Two Lane) Foot path : 1m on either side Width of bearing : 400 mm Wearing coat : 80mm average Loading : IRC Class AA (Tracked) Materials : M30 concrete and Fe 415 Steel |
| --- | --- |
| 2. | Design a RCC Tee beam girder bridge for the following data: Clear width of road way : 7.5 m Span (Centre to centre of bearing) : 16 m Live load : IRC class AA Tracked vehicle Average thickness of wearing coat : 80 mm No. of main girders : 4 Spacing of cross girders : 4m Materials : M30 concrete and Fe 415 Steel |
| 3. | Design a solid slab bridge for the following data: Clear distance between abutments : 9m Road : NH (Two Lane) Foot path : 1m on either side Width of bearing : 400 mm Wearing coat : 80mm average Loading : IRC Class A (Tracked) Materials : M30 concrete and Fe 415 Steel |
| 4. | Design a solid slab bridge for the following data: Clear distance between abutments : 6m Road : NH (Two Lane) Foot path : 1m on either side Width of bearing : 400 mm Wearing coat : 80mm average Loading : IRC 70R (wheeled) Materials : M30 concrete and Fe 415 Steel |
| 5. | Design a RCC Tee beam girder bridge for the following data: Clear width of road way : 8.5 m Span (Centre to centre of bearing) : 20 m Live load : IRC class A Tracked vehicle Average thickness of wearing coat : 80 mm No. of main girders : 4 Spacing of cross girders : 4m Materials : M30 concrete and Fe 415 Steel |
| 6. | A reinforced concrete simply supported slab is required for the deck of a road bridge having the data given below: Width of carriageway - 7.5 m Width of kerb – 600 mm Clear span – 5m Width of bearing – 400 mm Type of loading I.R.C. class AA or A whichever gives the worst effect. Materials : M30 concrete and Fe 415 Steel |
| 7. | A reinforced concrete simply supported slab is required for the deck of a road bridge having the data given below: Width of carriageway - 9.5 m Width of kerb – 600 mm Clear span – 5m Width of bearing – 400 mm Type of loading I.R.C. class A. Materials : M30 concrete and Fe 415 Steel |

Unit –II

Part-A

| 1. | What are the advantages of continuous bridges? |
| --- | --- |
| 2. | How will you find the radius and thickness of arch bridges? |
| 3. | What is called Articulation? |
| 4. | How will you find Bending Moment in slabs spanning in two directions. |
| 5. | How will you find reaction factor for guider using Courbon’s Theory? |
| 6. | How the concentrated load is calculated in Box culvert. |
| 7. | State the types of loading considered in box culverts. |
| 8. | Draw the neat sketch of balanced cantilever bridges. |
| 9. | In what situations balanced cantilever bridges are considered. |

Part-B

| 1. | Write the step by step procedure of design of long span continuous bridge. |
| --- | --- |
| 2. | Sketch the typical cross section of Balanced Cantilever Bridge. What are the advantages of this construction? |
| 3. | Explain the neat procedure of design of Arch Bridges. |
| 4. | Explain in detail about the design of Box Culvert. |
| 5. | Write the step by step procedure of design of box girder bridges. |
| 6. | Explain in detail about the types of loading considered in Box culvert. |
| 7. | Explain the method of designing a slab deck in integrated with longitudinal and cross section girder using Pigeauds method to support live loads due to IRC Class AA tracked vehicle. |

Unit III

Part-A

| 1. | What are the span range for solid and Tee beam bridges? |
| --- | --- |
| 2. | What are the advantages of Prestressed concrete bridges? |
| 3. | Draw any two cross sections for the pre- tensioned concrete bridges? |
| 4. | Draw any two cross sections for the post- tensioned concrete bridges? |
| 5. | What is meant by maximum and Minimum prestressing forces? |
| 6. | How will you find the eccentricity of cables in prestressed concrete bridges? |
| 7. | Define short term and long term deflection. |
| 8. | Explain the procedure for end block design. |

Part-B

| 1. | Design a post tensioned prestressed concrete slab bridge deck to suit the following data: Clear span : 12m Width of bearing : 400 mm Clear width of road way : 7.5m Foot path : 1m on either side Kerb : 600 mm wide Thickness of wearing coat : 80 mm Loading : IRC Class AA tracked vehicle Compressive strength at Transfer : 35 N/mm2 Materials: M40 grade concrete and 7 mm diameter high tensile wires with an ultimate strength of 1500 N/mm2. For supplementary reinforcement adopt Fe415 grade steel bars. |
| --- | --- |
| 2. | Design a post tensioned prestressed concrete T- beam and slab bridge deck to suit the following data: Effective span : 24 m Width of carriage way :7.5 m Kerbs 600 mm wide on either side of the road Spacing of main girders : 2m Spacing of cross girders : 4m Loading is IRC Class AA tracked vehicle Adopt M-45 Grade concrete and High tensile steel strands conforming to IS: 6006 and supplementary reinforcement comprising Fe 415 Grade HYSD bars. Permissible stresses are as specified in IRC: 18- 2000. Loss ratio = 0.85 |
| 3. | Design a post tensioned prestressed concrete T- beam and slab bridge deck to suit the following data: Effective span : 30 m Width of carriage way :7.5 m Kerbs 600 mm wide on either side of the road Spacing of main girders : 2m Spacing of cross girders : 4m Loading is IRC Class AA tracked vehicle Adopt M-45 Grade concrete and High tensile steel strands conforming to IS: 6006 and supplementary reinforcement comprising Fe 415 Grade HYSD bars. Permissible stresses are as specified in IRC: 18- 2000. Loss ratio = 0.85 |
| 4. | Design a post tensioned prestressed concrete slab bridge deck to suit the following data: Clear span : 10m Width of bearing : 400 mm Clear width of road way : 7.5m Foot path : 1m on either side Kerb : 600 mm wide Thickness of wearing coat : 80 mm Loading : IRC Class A tracked vehicle Compressive strength at Transfer : 35 N/mm2 Materials: M40 grade concrete and 7 mm diameter high tensile wires with an ultimate strength of 1500 N/mm2. For supplementary reinforcement adopt Fe415 grade steel bars. |
| 5. | Design a post tensioned prestressed concrete T- beam and slab bridge deck to suit the following data: Effective span : 25 m Width of carriage way :7.5 m Kerbs 600 mm wide on either side of the road Spacing of main girders : 2m Spacing of cross girders : 4m Loading is IRC Class A tracked vehicle Adopt M-45 Grade concrete and High tensile steel strands conforming to IS: 6006 and supplementary reinforcement comprising Fe 415 Grade HYSD bars. Permissible stresses are as specified in IRC: 18- 2000. Loss ratio = 0.85 |

Unit –IV

Part-A

| 1. | What are the span range for solid and Tee beam bridges? |
| --- | --- |
| 2. | What are the advantages of plate girder bridges? |
| 3. | How will you find the economical depth of the plate Girder? |
| 4. | Explain the loads considered in Railway bridges. |
| 5. | Draw the neat sketches of truss bridges |
| 6. | What is meant by maximum and Minimum prestressing forces? |
| 7. | What is purposes of providing Lateral Bracings in plate girder bridges. |

Part-B

| 1. | Design a deck type welded plate girder bridge to suit the following data: Effective span of the girder – 30 m Dead load – 7.5 kN/m Equivalent total live load for bending moment calculation/track – 2727 kN Equivalent total live load for shear calculation/track – 2927 kN |
| --- | --- |
| 2. | Design a deck type welded plate girder bridge to suit the following data: Effective span of the girder – 30 m Dead load – 7.5 kN/m |
| 3. | Design a plate girder to carry a super imposed load of 180 kN/m on an effective span of 15 m. |
| 4. | Design a steel trussed bridges to suit the following data: Effective span – 30m Roadway -7.5m (two lane) Kerbs -600mm Loading : IRC class AA tracked vehicle Materials : M25 Grade concrete and Fe415 HYSD bars for deck slab. Rolled steel sections with an yield stress of 236 N/mm2. |
| 5. | Design a steel trussed bridges to suit the following data: Effective span – 33m Roadway -7.5m (two lane) Kerbs -600mm Loading : IRC class AA tracked vehicle Materials : M25 Grade concrete and Fe415 HYSD bars for deck slab. Rolled steel sections with an yield stress of 236 N/mm2. |